Digital & Control System Component Specifications

1) OR Gate (74HC32 Type)

Electrical Specifications

- Logic Family: CMOS (74HC series)

- Supply Voltage (Vcc): 2V – 6V

- Typical Operating Voltage: 5V

- Input Voltage (High): ≥3.5V (at 5V Vcc)

- Input Voltage (Low): ≤1.5V (at 5V Vcc)

- Output Current: ±25 mA (max)

- Propagation Delay: ~8 ns

- Operating Temperature: -40°C to +85°C

Mechanical Specifications

- Package: DIP-14 (Through-hole)

- Pin Pitch: 2.54 mm

- Body Width: 7.62 mm

2) XOR Gate (74HC86 Type)

Electrical Specifications

- Logic Family: CMOS (74HC series)

- Supply Voltage (Vcc): 2V – 6V

- Typical Operating Voltage: 5V

- Output Current: ±25 mA

- Propagation Delay: ~8 ns

- Operating Temperature: -40°C to +85°C

Mechanical Specifications

- Package: DIP-14

- Pin Pitch: 2.54 mm

- Body Width: 7.62 mm

3) Flip-Flop (74HC74 D Flip-Flop)

Electrical Specifications

- Type: Dual D-Type Positive Edge Triggered

- Supply Voltage: 2V – 6V

- Typical Voltage: 5V

- Propagation Delay: ~13 ns

- Output Current: ±20 mA

- Setup Time: ~20 ns

- Operating Temperature: -40°C to +85°C

Mechanical Specifications

- Package: DIP-14

- Pin Pitch: 2.54 mm

- Body Width: 7.62 mm

4) ATmega328P

Electrical Specifications

- Architecture: 8-bit AVR RISC

- Operating Voltage: 1.8V – 5.5V

- Typical DIY Voltage: 5V

- Clock Frequency: Up to 20 MHz

- Flash Memory: 32 KB

- SRAM: 2 KB

- EEPROM: 1 KB

- ADC: 10-bit

- I/O Pins: 23 programmable

- Operating Current: ~10 mA

Mechanical Specifications

- Package: DIP-28

- Pin Pitch: 2.54 mm

- Operating Temperature: -40°C to +85°C

5) Resistors (10kΩ Pull-up Resistors – ¼ Watt)

Electrical Specifications

- Resistance Value: 10 kΩ

- Power Rating: 0.25W

- Tolerance: ±5%

- Maximum Working Voltage: 250V

- Temperature Coefficient: ±200 ppm/°C

Mechanical Specifications

- Type: Carbon Film

- Body Length: ~6.3 mm

- Lead Diameter: ~0.6 mm

- Mounting: Through-hole

6) 5V Relay (SPDT)

Electrical Specifications

- Coil Voltage: 5V DC

- Coil Current: ~70 mA

- Contact Rating: 10A @ 250V AC

- Contact Type: SPDT

Mechanical Specifications

- PCB Mount (Through-hole)

- Dimensions: ~19 × 15.5 × 15 mm

- Operating Temperature: -40°C to +85°C

7) BC547 (NPN Transistor)

Electrical Specifications

- Collector-Emitter Voltage: 45V

- Collector Current: 100 mA (max)

- Power Dissipation: 500 mW

- DC Gain (hFE): 110 – 800

Mechanical Specifications

- Package: TO-92

- Mounting: Through-hole

- Operating Temperature: -55°C to +150°C

8) Buzzer 5V (Active Type)

Electrical Specifications

- Rated Voltage: 5V DC

- Operating Voltage: 3V – 7V

- Current Consumption: 20 – 30 mA

- Sound Level: 85 – 95 dB

- Frequency: ~2 kHz – 4 kHz

Mechanical Specifications

- Type: PCB mount

- Diameter: 12 – 16 mm

- Height: 7 – 10 mm

- Pin Pitch: 5 mm

9) FR4 PCB

Electrical Specifications

- Dielectric Constant: 4.2 – 4.8

- Breakdown Voltage: ~20 kV/mm

- Insulation Resistance: >10⁶ MΩ

- Operating Temperature: -50°C to +130°C

Mechanical Specifications

- Thickness: 1.6 mm (standard)

- Copper Thickness: 35 µm (1 oz)

- Layers: 1–2 layer

- Flame Rating: UL94-V0

10) ACS Current Sensor (ACS712 Type)

Electrical Specifications

- Supply Voltage: 5V

- Current Range Options: ±5A / ±20A / ±30A

- Sensitivity:

  - 185 mV/A (5A version)

  - 100 mV/A (20A version)

  - 66 mV/A (30A version)

- Output Voltage: Vcc/2 at 0A

- Bandwidth: 80 kHz

- Isolation Voltage: 2.1 kV RMS

Mechanical Specifications

- Module Size: ~31 × 13 mm

- Mounting: Through-hole module

- Terminal Type: Screw terminal

- Operating Temperature: -40°C to +85°C

11) ZMPT101B Voltage Sensor Module

Electrical Specifications

- Input Voltage: 0 – 250V AC

- Output: Analog voltage proportional to AC input

- Supply Voltage: 5V DC

- Isolation: Transformer-based isolation

- Adjustable Gain: Via onboard potentiometer

- Frequency Range: 50/60 Hz

Mechanical Specifications

- Module Size: ~49 × 19 mm

- Mounting: PCB module

- Terminal Type: Screw terminal

- Operating Temperature: -40°C to +85°C